Technical rider

Thelma&Clyde

Sound

General information:

In case of problems meeting this specification, please do not hesitate to contact the artist through your contact. We ask local promoters or sound engineers not to make changes in this spec, on behalf of the artist, without confirming with artist or representative. Brands and types of sound equipment is meant as quality guidance, and not as absolute requirements if not stated specifically. The artist will have absolute artistic control during the performance. Please inform in advance of local rules and regulations regarding sound level and use of other effects.

Equipment quality:

Sound equipment must be of high standard and free from audible noise. Mixing console must be in a good position in the audience area, centered between left and right PA to ensure good control possibilities. Do not place on or under balcony and not on any platform or podium if possible.  Sound system must be rigged and tested for functionality before artist arrives for setup and sound check. Local system engineer must be available for the artists own crew at all times from sound check to end of concert.

Sound System:

High quality 3 or 4 way active PA-system. The system must cover all audience areas with uniform and

undistorted sound. It is important that the system is free of hum and noise. (pref: EAW 750/850, d&B,

Nexo, Meyer, Turbosound or similar). Please note that this music tends to be highly compressed and powerful.  Make sure the PA has enough power for this type of music. The artist will not be responsible for any damage   done to the equipment due to under powering or under scaling.

FOH:

A Minimum 32 channels (24 mono), with 8 auxiliaries, minimum 5 pre-fader.

4 band input eq (min. 2sweep mid filters)

48v, phase switch, insert and high pass filters on all inputs

(Pref: Midas, DDA, Soundcraft or similar)(Not Spirit or equal)

Please no digital console!

Outboard:

Min 1 digital reverb unit. (pref: PCM91/90/80/70, TC M-5000/3000)

1 digital delay unit with tap. (min 1 sec) (pref: TC 2290/D-TWO)

1 multi fx unit. (pref: SPX 990/90II/2000)

Min 4 ch. compressor, pref. 8 ch.

Min 2 ch. gate, pref. 4 ch.

Graphic EQ’s on all speaker feeding outputs

1 CD-player

Monitor System:

Minimum 6 wedges on min. 5 separate mixes from FOH or monitor mix. (Artist does NOT bring monitorengineer)

31 bands Equalizer pr mix.

Some giggs will only require 4 mixes, please check with your contact in advance.

Microphones/stands:

| CH | Instrument | Mic | Stand | Patch | Insert | +48V |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Kick | Shure beta91 | - |  | gate1 | x |
| 2 | Kick | Shure beta52 | small |  | gate2 |  |
| 3 | snare 1 | Shure sm57 | small |  | comp1 |  |
| 4 | snare 2 | Shure sm57 | small |  | comp2 |  |
| 5 | Hihat | Shure sm81 | small |  |  | x |
| 6 | floor tom | Sennheiser 904 | clip |  | gate3 |  |
| 7 | overhead L | Shure sm81 | small |  |  | x |
| 8 | overhead R | Shure sm81 | small |  |  | x |
| 9 | Bass | Active DI | - |  |  | x |
| 10 | Bass | Shure sm57 | small |  |  |  |
| 11 | bass synth. L | Active DI | - |  |  | x |
| 12 | bass synth. R | Active DI | - |  |  | x |
| 13 | key 1 L | Active DI | - |  |  | x |
| 14 | key 1 R | Active DI | - |  |  | x |
| 15 | key 2 L | Active DI | - |  |  | x |
| 16 | key 2 R | Active DI | - |  |  | x |
| 17 | Omnichord | Active DI | - |  |  | x |
| 18 | tracks/mac L | Active DI | - |  |  | x |
| 19 | tracks/mac R | Active DI | - |  |  | x |
| 20 | vocal Maylen | Shure beta58 | large |  | comp3 |  |
| 21 | vocal Hanne | own | large |  | comp4 |  |
| 22 | vocal Hanne fx | Active DI | - |  |  | x |
| 23 | vocal Morten | Shure beta58 | large |  | comp5 |  |
| 24 | vocal Trygve | Shure beta58 | large |  | comp6 |  |

Contact:

Sound: Ruben Lervåg, [+47] 916 88 567, ruben@lervag.no
Booking/management: Jan K. Transeth, [+47] 906 10 711, transitjan@gmail.com